# NCP4 Metering Walkdown and Tag Review

Plant: Northline Components Plant 4
Inspection date: 08 June 2026
Prepared by: Facilities Reliability Engineering
Purpose: Field-verify compressed-air equipment configuration, historian tags, pressure-measurement locations, and unresolved engineering items before retrofit approval.

# 1. Evidence-Handling Note

This walkdown records field observations only. Field-verified nameplates and installed controls supersede stale register descriptions for current equipment configuration. A signed maintenance record is required to change equipment availability. Flowmeter scaling must be reconciled against the separate calibration workbook before logger values are used.

# 2. Installed Equipment Verification

| Asset | Field observation | Conflict or status | Engineering treatment |
| --- | --- | --- | --- |
| C-101 | Motor nameplate: 110 kW. Local panel: fixed-speed load/unload. | Matches asset register. | Use as available base-load unit. |
| C-102 | Motor nameplate: 75 kW. ABB ACS880 variable-speed drive fitted. Local HMI shows 500-1,200 Nm3/h trim range. | Conflicts with register entry of 90 kW fixed-speed compressor. | Use field verification with maintenance record. Do not use the old 90 kW load-percentage calculation as a power meter. |
| C-103 | Unit isolated and tagged OUT OF SERVICE. Oil residue visible downstream of separator housing. | Conflicts with register status 'available'. | Use maintenance history to determine availability. Do not count C-103 as operating or commissioning backup capacity. |
| DR-301 / F-301 | Dryer and filter train remain in service. Differential pressure alarm observed during high-flow event. | Register lists only clean-element pressure drop. | Use as-found peak differential pressure for current pressure reconstruction; use clean serviced value only for post-retrofit scenario. |
| V-401 | Receiver nameplate indicates 6.0 m3 geometric volume. | Matches register. | Treat 6.0 m3 as existing geometric storage volume. |

# 3. Historian and Transmitter Tag Review

| Tag | As-found configuration | Conflict | Engineering treatment |
| --- | --- | --- | --- |
| FT-201_MAIN_FLOW | Historian configured for 0-1,800 Nm3/h at 4-20 mA. | Replacement transmitter plate indicates 0-2,400 Nm3/h. | Historian flow export is under-scaled. Apply the correction factor recorded in the calibration workbook. |
| C102_LOAD_PCT | Derived tag multiplies drive loading percentage by legacy 90 kW motor rating. | Installed motor is 75 kW and VSD panel exposes direct kW measurement. | Reject derived legacy power tag. Use C102_DRIVE_KW. |
| PT-401_HEADER | Receiver discharge header pressure at compressor room. | Valid sensor location. | Use as upstream pressure only; it is not the packaging-hall point-of-use pressure. |
| PT-404_PACK_POI | Packaging line point-of-use pressure downstream of treatment, ring main, and local regulator. | Critical sensor confirmed at packaging manifold. | Use to judge the 6.30 bar(g) packaging requirement. |

# 4. Simultaneous Peak-Event Pressure Walkdown

A short packaging-hall high-demand event was observed at 14:17:20. Values below are simultaneous field readings cross-checked against the logger workbook.

| Measurement point | Observed pressure | Interpretation |
| --- | --- | --- |
| PT-401 receiver header | 6.95 bar(g) | Upstream reference. |
| Dryer/filter outlet gauge | 6.60 bar(g) | 0.35 bar drop across DR-301/F-301 as found. |
| Packaging-hall ring-main inlet | 6.32 bar(g) | Additional 0.28 bar distribution drop. |
| PT-404 packaging manifold | 6.16 bar(g) | Additional 0.16 bar local regulator/branch drop; below required 6.30 bar(g). |

# 5. Post-Service Pressure-Drop Basis for Retrofit Screening

For post-retrofit screening only, facilities engineering authorizes the following maintained pressure-drop assumptions at the corrected peak flow: 0.18 bar across serviced dryer/filter equipment, 0.25 bar through the distribution route, and 0.12 bar through the local regulator and branch connection. The retrofit must maintain at least 6.30 bar(g) at PT-404.

# 6. Unresolved Items Requiring Follow-Up

The receiver-pad structural drawing and anchor-bolt verification for a new receiver are not present in the supplied package.

The installed master-sequencer firmware version has not been checked against the proposed C-102 VSD integration module.

The final safety-valve selection and statutory inspection plan for added receiver capacity are not included.

A potential future packaging-line expansion is mentioned informally but has no approved demand profile or project authorization.

# 7. Domain Reasoning Notes for the Reviewer

Do not use compressor-room header pressure as a substitute for the critical point-of-use pressure.

Do not combine distribution leakage and dryer purge loss into one loss category.

Do not treat a stale asset register as authoritative when later field and maintenance records establish the installed configuration.

Do not count an unavailable compressor toward reliable capacity or commissioning backup.